The microscope

The microscope, sometimes called the compound microscope, has two short focal length convex lenses. The objective gives a real, inverted image of the object which is then magnified by the eye lens, giving a final virtual image. In many professional microscopes both the eye lens and the objective lens are made of two or more lenses fitted together. 

In a microscope the object is slightly further from the objective lens than its focal length. Shorter focal length objectives give higher magnifications and this means that the object will be very close to the objective lens.

Some microscopes even have oil between the lens and the object to help give high quality magnification.

The final image is usually at a distance of about 25 cm from the eye, if it was closer than this you would get eye strain if you tried to use the microscope for a long time.

With normal microscopes you can get magnifications up to about 500 times but using an electron microscope, that works in a quite different way, it is possible to magnify objects by over 100 000 times!

  

In Figure 1 you can see how the objective lens makes an initial real image of the object which is then magnified using the eye lens to give a final enlarged image of the object.

Figure 2 shows the outside of a microscope.

In the ray diagram the sizes of both lenses, particularly the eye lens have been exaggerated to make the diagram easier to follow.





objective lens





eye lens





final image





initial image





object





eye





Figure 1





Figure 2








schoolphysics


