# The law of moments

AIM:

Load 1

Load 2

pivot

d1

d2

The aim of this experiment is to confirm the law of moments

YOU WILL NEED:

Two sets of slotted masses (0-1 kg in 100g)  String  A metre ruler with a hole drilled at its centre  A retort stand , boss and clamp  A pivot such as a nail (careful with the sharp end) A little plasticene

WHAT TO DO:

Set up the apparatus as shown in the diagram. Hang a load on one side of the pivot and balance it by hanging a second load the other side of the pivot and moving this load until your ruler is horizontal again.

| Load 1 (N) | Distance of load 1 from pivot (cm) (d1) | Load 2 (N) | Distance of load 2 from pivot (cm) (d2) |
| --- | --- | --- | --- |

RESULTS:

Copy and complete a table of results like the one below.

ANALYSIS AND CONCLUSION:

For each line of your table work out:

Load 1 x distance of load 1 from pivot and load 2 x distance of load 2 from pivot

Try and suggest a law relating the size and position of the loads using those two numbers.