The Cathode Ray Oscilloscope

Basic operation.

Before doing the experiments you should adjust the controls for focus, brilliance, X and Y shift until you have a dot in the middle of the screen.

Fluorescent screen

Y plates

X plates

Electron gun

anode

cathode

electron beam

The simplified diagram above shows the inside of a cathode ray oscilloscope.

Copy the diagram into your books.

These experiments will help you understand how a cathode ray oscilloscope works.

You will need a CR0, a battery pack, a high current power supply, and a microphone and leads.

Instructions.

Switch on your oscilloscope and adjust the controls to give a straight line across the centre of the screen.

The Y gain should be set at 1V/division and the time base set at 1 ms/division,

1. Connect one 1.5v cell to the Y inputs and adjust the Y gain so that the line moves up 1.5 divisions.

AFTER THIS DO NOT ALTER THE Y SHIFT OR Y GAIN.

2. Now connect two cells and then three cells. Record the line deflection each time.

3. Reverse the leads and repeat number two.

4. Now connect the 1V AC supply from the high current power supply to the Y inputs.

Record the size and shape of the trace. Repeat with the 2V AC input and then the DC input, recording the trace each time. You must have steady trace for each experiment.

5. With the 2V AC connected try increasing the Y gain. Then try varying the-variable control-. Record the results.

6. Now connect a microphone to the Y inputs and record the effect of talking into it.