# Critical potentials of helium gas

AIM:

The aim of this complex experiment is to show experimental evidence for the energy level structure of an atom. Helium gas is used for this and a full experimental account may be found in the booklet supplied with the tube.

YOU WILL NEED:

Critical potentials tube and stand  	Low voltage (6-7V a.c) supply for heater

30 V supply for anode  3 1.5 V dry cells  Leads  Spot galvanometer  Milliammeter (0-10 mA)

G

30 V d.c

3, 4.5 or 6 V d.c

6V a.c

Collector current

Anode voltage

WHAT TO DO:

Set up the apparatus as shown in the diagram.

Switch on the heater supply and adjust it to 6 V. Switch on the anode supply and slowly increase it, monitoring the anode current as you do so. Record the value of the collector current using the spot galvanometer.

A series of peaks and troughs should be obtained as shown in the accompanying graph.

ANALYSIS AND CONCLUSION:

NOTE: THIS IS VERY EXPENSIVE APPARATUS. PLEASE HANDLE WITH CARE AND CHECK WITH THE STAFF BEFORE TURNING ON!