# FUSING CURRENTS IN WIRES

AIM:

The aim of this experiment is to investigate the current at which a piece of wire will break.

YOU WILL NEED:

Power supply, leads, crocodile clips or other suitable wire holder, wires, ammeter, a ruler, a micrometer screw gauge

WHAT TO DO:

Set up a circuit that will enable you to measure the current through a specimen of wire. Measure the diameter of a single piece of wire and then connect it between the crocodile clips. SLOWLY increase the current until the wire breaks.

Record this current.

Repeat the experiment for a number of different lengths of wire, a number of different types of wire and also some wire wool.

Now test the fusing current when a current close to that of the previously measured fusing current is suddenly applied.

Plot a graph of fusing current against wire length.