Electricity charge and current

(If needed take the charge on the electron to be - 1.6 x1019 C)

1.	Why does the resistance of a metal increase with temperature?

2.	(a) What is meant by free electrons?

(b)	how are they moving within a solid?

3.	Why does an insulator not conduct electricity?

4.	What would you have to do to a sample of semiconductor to decrease its resistance? Explain your answer.

5.	What is the approximate drift velocity of a ‘cloud’ of free electron through the bulb in a torch?

6.	Express the following in AMPS:

(a) 2mA	(b) 5A	(c) 6MA	(d) l2pA	(e) 102mA

7.	How could the drift velocity of electons be increased?

8.	Draw a diagram of a Thermionic diode and explain how the number of electrons crossing the tube varies with the applied voltage.

9.	Calculate the charge passing a point in a wire when the following currents flow in a wire for the times given:

(a) 25mA 20s (b) 3A 5m (c) 10 A 5 weeks

10. Calculate the current in a wire when the following charges flow past in the times given:

(a)	20 C in 5s

(b)	0.5 C in 25

(c)	1015 electrons in 2s

(d)	12 C in 2 minutes