BENDING OF A BEAM

L

W

AIM:

The aim of this experiment is to investigate the factors affecting the bending of a uniform horizontal beam.

YOU WILL NEED:

A set of slotted masses (0-1kg), a pair of knife edges, two retort stands and bosses, a ruler, a base clamp, a suitable beam such as another metre rule, string or thread

WHAT TO DO:

Set up the beam on two knife edges as shown and load the beam at its centre.

Make two sets of measurements:

(i)	record the depression (y) for various loads (W = Mg) with constant length (L)

(ii)	record the depression (y) for various lengths (L) with a constant load (W)

ANALYSIS AND CONCLUSIONS:

Plot two graphs

(i)	Y against W	(ii) lg y against lg L

From your graphs determine a possible equation for the bending of the beam.

# FURTHER WORK

Calculate the Young modulus (E) for the beam from the following equation:

Young modulus (E) = [L2g/4bd3][M/y]

For this you will need to measure the thickness (d) and the width (b) of the beam. Use a pair of vernier calipers or a micrometer screw gauge for this.