Moments and couples

Take 9 = 9.8 ms-2 where needed

1. What is meant by the moment of a force?  Use a diagram to help illustrate your answer.

2. What is meant by a couple?

3. What is meant by the moment of a couple or torque?

4. Write down the principle of moments.

5. Give two practical examples of the application of the principle of moments.

6. A door has a handle which is 0.8 m from the hinge. What is the moment of the following forces applied to the handle about the hinge:

(a) 12 N at right angles to the door

(b) 20 N at 70o to the door

60o

70 kg

W

5 m

3 m

Diagram 1

(c) 100 N at 25o to the door

7. A mass of 10 kg hangs from one end of a 1 m long light rod that is pivoted 0.3 m from that end.

(a) what force must be applied at the 0.6 m mark to balance the rod

(b) if  force of 20N is hung from the 0.5 m mark what force must be hung from the 1 m mark to balance the rod?

8. What is the counterweight (W) required to balance the light crane arm shown in diagram 1?

20 N

3 m

1.5 m

T

70o

Diagram 2

9.What is the tension (T) in the cable shown in  diagram 2?

T

25 m

20 m

65o

70 N

Diagram 3

10. What is the tension (T) in the cable shown in diagram 3?